Table of Contents

# CAN Bootloader Console

A cross-platform console application for updating firmware on CAN (Controller Area Network) bus devices using a custom bootloader protocol.

## Overview

This application provides a robust, interactive interface for scanning CAN bus devices, managing firmware versions, and performing over-the-air firmware updates on embedded devices. It supports both online (database-connected) and offline (cache-only) operation modes.

## Features

- Interactive Menu System: User-friendly console interface with three operator modes (Simple, Advanced, Debug)

- Device Scanning: Automatic discovery of CAN devices on the bus with serial number and firmware version detection

- Firmware Management:

  - Database integration with MySQL for centralized firmware repository

  - Local encrypted firmware cache for offline operation

  - Multi-version firmware support per product

- Firmware Upload:

  - Reliable firmware update process with automatic retry logic

  - Flash erase and write operations

  - Bootloader entry and application restart

- Cross-Platform Support: Windows, macOS (Intel & ARM64)

- Multi-Device Support: Handles devices with bridge boards (up to 5 boards per device)

## System Requirements

### Runtime Requirements

- .NET 8.0 Runtime (included in self-contained deployment)

- Serial Port access for CAN communication

- Network Connection (optional, for database access)

### Build Requirements

- .NET 8.0 SDK

- Visual Studio 2022 or VS Code with C# extension

## Installation

### Pre-built Releases

Download the appropriate release for your platform:

#### Windows (x64)

bin/Release/net8.0/win-x64/publish/CANBootloaderConsole.exe

#### macOS (ARM64)

bin/Release/net8.0/osx-arm64/publish/CANBootloaderConsole
# Or use the launcher script:
./bin/Release/net8.0/osx-arm64/publish/Start CANBootloader.command

### Building from Source

# Clone the repository
git clone <repository-url>
cd CANBootloader_Core

# Build for current platform
dotnet build -c Release

# Or publish for specific platform
dotnet publish -c Release -r win-x64 --self-contained
dotnet publish -c Release -r osx-arm64 --self-contained

## Usage

### Quick Start

- Launch the application:

- # Windows
CANBootloaderConsole.exe

# macOS
./CANBootloaderConsole

- Select Operator Mode (press M):

  - Simple: Streamlined interface for basic firmware updates

  - Advanced: Standard operations with detailed status information

  - Debug: Verbose logging for troubleshooting

- Update Firmware (Simple Mode):

  - Press 1 to start firmware update

  - Follow the on-screen prompts

### Operator Modes

#### Simple Mode

- Minimal output

- Direct firmware update workflow

- Ideal for production use

#### Advanced Mode

Adds access to: - Device scanning - Cache management - Firmware browsing - COM port listing

#### Debug Mode

- Verbose CAN message logging

- Detailed communication traces

- Retry attempt notifications

- Ideal for troubleshooting

### Main Menu Options

Simple Mode: - 1 - Update Firmware - M - Change Operator Mode - Q - Quit

Advanced/Debug Modes: - 1 - Scan for Devices - 2 - Update Firmware - 3 - Update Cache from Database - 4 - Show Cached Firmware - 5 - List COM Ports - M - Change Operator Mode - Q - Quit

## Architecture

### Core Components

#### 1. Program.cs

- Application entry point

- Console initialization (disables QuickEdit mode on Windows)

- Firmware cache and repository initialization

- Error handling and logging

#### 2. InteractiveMenu.cs

- Main user interface loop

- Menu navigation and command routing

- Device discovery workflow

- Firmware selection and validation

#### 3. FirmwareUploader.cs

- Firmware upload orchestration

- Bootloader mode entry

- Bridge board communication

- Upload progress tracking and retry logic

#### 4. BootloaderProtocol.cs

- State machine for bootloader communication

- HEX file parsing and packet generation

- Flash erase and write commands

- Timeout handling with automatic retries (up to 5 attempts)

#### 5. CANPort.cs

- Serial port wrapper for CAN communication

- Message parsing and formatting

- Event-driven message reception

- Platform-specific serial port configuration

#### 6. DeviceScanner.cs

- CAN bus device discovery

- Serial number and firmware version querying

- Bridge board detection

- Device information aggregation

#### 7. FirmwareRepository.cs

- MySQL database integration

- Firmware version queries

- Product and board configuration retrieval

- Multi-version firmware support

#### 8. FirmwareCache.cs

- Local firmware storage with encryption

- Cache index management (JSON)

- File migration from unencrypted to encrypted format

- Cache update and synchronization

### Data Models

#### FirmwareInfo

class FirmwareInfo
{
    string PartNumber;        // Product part number (e.g., "200-XXXX")
    byte BridgeId;            // Bridge board identifier (1-5)
    int Version;              // Numeric version (e.g., 110032073)
    string VersionString;     // Human-readable version (e.g., "v11.32.73")
    string HexFilePath;       // Path to cached HEX file
    byte[] HexFileData;       // Binary HEX file content
    DateTime LastUpdated;     // Cache timestamp
}

#### CANDevice

class CANDevice
{
    byte CanId;              // CAN bus identifier
    ulong SerialNumber;      // Device serial number
    string SerialFormatted;  // Formatted serial (e.g., "12-345678-9")
    byte DeviceId;           // Product identifier
    string PartNumber;       // Product part number
    int FirmwareVersion;     // Current firmware version
    string FirmwareString;   // Formatted version string
}

## CAN Protocol

### Message Structure

The application uses J1939-style CAN messaging:

CAN Identifier Format (29-bit):

Byte 0: Priority/PGN (MSB)
Byte 1: PGN
Byte 2: Destination Address
Byte 3: Source Address (PCID)

Bootloader CAN ID: 0x19FF22

### Key Protocol Commands

#### Enter Bootloader

ID:  [0x19, 0xEF, <canId>, 0xFE]
MSG: [0x00, 0x06, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00]

#### Request Serial Number

ID:  [0x18, 0xEA, 0xFF, 0xFE]
MSG: [0x01, 0xFF, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00]

#### Request Firmware Version

ID:  [0x18, 0xEA, <canId>, 0xFE]
MSG: [0x02, 0xFF, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00]

### Bootloader State Machine

- RequestInfo: Query bootloader version and device information

- GenerateHexData: Parse HEX file

- EraseFlash: Erase application flash memory

- HexDataToPack: Split firmware into CAN-sized packets (7 bytes)

- WriteData: Transmit firmware packets

- WriteDataLast: Send final packet

- GoToApp: Reset device to application mode

- Cleanup: Finalize and disconnect

## Configuration

### Database Connection

Edit Program.cs to configure MySQL connection:

private const string SQL_CONNECTION_STRING = 
    "Server=<host>;Database=<db>;User Id=<user>;Password=<pass>;Port=3306;SslMode=None;";

### Cache Location

Default cache directory: - Windows: %AppData%\CANBootloader\FirmwareCache - macOS/Linux: ~/.config/CANBootloader/FirmwareCache

### Timeout Configuration

Windows: 1 second communication timeout
macOS/Linux: 3 seconds (for slower serial performance)

Modify in BootloaderProtocol.cs:

int timeoutMs = OperatingSystem.IsWindows() ? 1000 : 3000;

## Security

### Firmware Encryption

Cached firmware files are encrypted using CacheEncryption.cs: - Algorithm: AES-256 (or similar, depending on implementation) - File Extension: .enc - Key Storage: Embedded in application - Migration: Automatic conversion from unencrypted legacy files

### Database Access

- Read-only database credentials recommended

- Connection uses MySQL over TCP (no SSL by default)

- Connection string stored in compiled binary

## Troubleshooting

### Common Issues

#### “Access denied to port”

- Cause: Serial port in use by another application

- Solution: Close other applications using the port, or change COM port

#### “Maximum retries reached”

- Cause: Communication timeout or device not responding

- Solution:

  - Verify CAN bus connection

  - Check baud rate (115200)

  - Switch to Debug mode for detailed diagnostics

  - Increase timeout on macOS/Linux

#### “SQL repository unavailable”

- Cause: Database connection failed

- Solution: Application continues in offline mode using cache. Verify:

  - Network connectivity

  - Database server availability

  - Connection credentials

#### “Device not entering bootloader mode”

- Cause: Device doesn’t respond to reset command

- Solution:

  - Verify device supports bootloader protocol

  - Check CAN ID is correct

  - Ensure device is powered and connected

### Debug Mode

Enable Debug mode (M -> 3) to see: - CAN message traces (TX/RX) - State machine transitions - Retry attempts - Detailed error messages

## Dependencies

### NuGet Packages

- System.IO.Ports (7.0.0): Serial port communication

- System.Management (8.0.0): Windows device management

- MySqlConnector (2.3.5): MySQL database access

- Newtonsoft.Json (13.0.3): JSON serialization

## Platform-Specific Notes

### Windows

- QuickEdit mode disabled automatically to prevent console freezing

- Standard serial port timeouts (100ms read, 500ms write)

- Uses System.Management for device enumeration

### macOS

- Extended timeouts for serial communication (3000ms)

- DTR/RTS disabled for better stability

- Additional buffer flush after port opening

- ARM64 support for Apple Silicon

- Startup script included: Start CANBootloader.command

### Linux

- Similar configuration to macOS

- Requires serial port permissions (add user to dialout group)

## Development

### Project Structure

CANBootloader_Core/
├── Program.cs                    # Entry point
├── InteractiveMenu.cs            # UI and workflow
├── FirmwareUploader.cs           # Upload orchestration
├── BootloaderProtocol.cs         # Bootloader state machine
├── CANPort.cs                    # Serial/CAN communication
├── FirmwareRepository.cs         # Database access
├── FirmwareCache.cs              # Local storage
├── CacheEncryption.cs            # Encryption utilities
├── OperatorMode.cs               # Mode definitions
├── Functions.cs                  # Helper utilities
├── ProtocolConstants.cs          # Protocol definitions
├── FirmwareInfo.cs               # Data models
├── UniFirmware.cs                # Firmware structures
├── BOOTHexDecoder.cs             # HEX file parser
├── DeviceScanner/
│   ├── DeviceScanner.cs          # Device discovery
│   └── CANDevice.cs              # Device model
└── CANBootloaderConsole.csproj   # Project file

### Building for Release

# Windows x64
dotnet publish -c Release -r win-x64 --self-contained -p:PublishSingleFile=true

# macOS ARM64
dotnet publish -c Release -r osx-arm64 --self-contained -p:PublishSingleFile=true

# macOS Intel
dotnet publish -c Release -r osx-x64 --self-contained -p:PublishSingleFile=true

### Code Style

- C# 10.0+ features

- Async/await pattern for I/O operations

- Event-driven CAN message handling

- Defensive programming with try-catch blocks

- Platform-specific conditionals using OperatingSystem class

## Contributing

### Guidelines

- Test on all supported platforms (Windows, macOS)

- Maintain backward compatibility with existing firmware cache

- Follow existing code style and patterns

- Update documentation for new features

- Use operator mode appropriately for new logging

### Testing Checklist

- Device scanning works on target platform

- Firmware upload completes successfully

- Retry logic handles timeouts gracefully

- Cache updates and persists correctly

- Database connectivity handles failures

- All operator modes display appropriate information

## License

[Add license information]

## Support

For issues, feature requests, or questions: - Create an issue in the repository - Contact: [Add contact information]

## Version History

[Add version history/changelog]

## Credits

Developed by: [Add developer/team information]
Organization: Clayton Power