The display can modify a parameter inside the LPS. To modify a parameter a menu parameter must be created as a setting. To create a setting a struct “SettingContainer” must be inserted as parameter into a menu.

Setting-related values

All settings use the same principle and are controlled by the same format/struct of data. Following table shows the data that controls the functionality of the setting.

STRUCT: SettingContainer

| Value | Datatype |
| --- | --- |
| STEP_FACTOR_SLOW | S32 |
| STEP_FACTOR_FAST | S32 |
| LIMIT_MIN | S32 |
| LIMIT_MAX | S32 |
| ULTIMATE_MIN_ENABLED | BOOL |
| ULTIMATE_MAX_ENABLED | BOOL |
| ULTIMATE_MIN | S32 |
| ULTIMATE_MAX | S32 |

NOTE: The menu is activated as a setting when LIMIT_MIN is not equal to LIMIT_MAX.

STEP_FACTOR_SLOW

When the user is changing the value inside the setting window it will increment in steps defined by STEP_FACTOR_SLOW.

STEP_FACTOR_FAST

When the user is changing the value inside the setting window by holding the increment button it will increment in steps defined by STEP_FACTOR_FAST.

LIMIT_MIN

The maximal lower value that the user can decrement to is defined by LIMIT_MIN

LIMIT_MAX

The maximal upper value that the user can increment to is defined by LIMIT_MAX

ULTIMATE_MIN / ULTIMATE_MIN_ENABLED

If value has reached LIMIT_MIN and ULTIMATE_MIN_ENABLED is set then the value of the menu parameter will automatically get the value contained by ULTIMATE_MIN.

ULTIMATE_MAX / ULTIMATE_MAX_ENABLED

If value has reached LIMIT_MAX and ULTIMATE_MAX_ENABLED is set then the value of the menu parameter will automatically get the value contained by ULTIMATE_MAX.

Settings

This section explains the setting which are implemented inside the display. The settings are following:

| Setting Name | Format |
| --- | --- |
| AC Output Time (Load Dep.) | TIME |
| Maximal AC Input Current | CURRENT |
| Jumpstart | TIME |
| DC Output Time | TIME |
| Language | LANGUAGE |

AC Output Time

AC Output Time defines the time the inverter will continue to run after the AC load is removed.

| Maximal Limit: | 10h |
| --- | --- |
| Minimal Limit: | 0s |
| Ultimate Max: | NONE |
| Ulitmate Min: | NONE |
| Slow Increment: | 10min |
| Fast Increment: | 1h |

Maximal AC Input Current

This value sets the upper limit for AC charging power. The value is representing the maximal current.

| Maximal Limit: | 5A |
| --- | --- |
| Minimal Limit: | 1A |
| Ultimate Max: | “Maximal” (5A) |
| Ulitmate Min: | NONE |
| Slow Increment: | 1A |
| Fast Increment: | 1A |

Jumpstart

Jumpstart is used to deliver DC power, for a short limit of time, through the DC-Out plug of the LPS.

| Maximal Limit: | 5min |
| --- | --- |
| Minimal Limit: | 0sec |
| Ultimate Max: | NONE |
| Ulitmate Min: | NONE |
| Slow Increment: | 1A |
| Fast Increment: | 1A |

DC Output Time

DC Output Time defines the time the DC Output will continue to run after the LPS is powered down.

| Maximal Limit: | 10h |
| --- | --- |
| Minimal Limit: | 0s |
| Ultimate Max: | NONE |
| Ulitmate Min: | NONE |
| Slow Increment: | 10min |
| Fast Increment: | 1h |

Language

Configure environment language

| Maximal Limit: | 1 |
| --- | --- |
| Minimal Limit: | 0 |
| Ultimate Max: | NONE |
| Ulitmate Min: | NONE |
| Slow Increment: | 1 |
| Fast Increment: | 1 |

0 = ENG

1 = DK