The display can support different type of date. This document describes the format.

Supported Data

The display has 3 main states (Popups, Menu and MainView) which support following formats:

| Format Index | Format |
| --- | --- |
| PREFIX_NONE | <I>,<F> |
| ERROR | <I>,<F> |
| VOLTAGE | <I>,<F>V |
| CURRENT | <I>,<F>A |
| POWER | <I>,<F>W |
| TEMPERATURE | <I>,<F>°C |
| PROCENT | <I>,<F>% |
| OPERATION | <STRING> |
| LANGUAGE | <STRING> |
| SERIAL | XXXXXX-XXXX |
| VERSION | XX.XX.XX |
| DATE | YYYY-MM-DD |
| TIME | HH:MM:SS / HH:MM |

PREFIX_NONE and ERROR

These types do not have a prefix. The value must be transmitted from the LPS to the display as a 32-bit signed value where the two LSB bytes are used as decimals. The maximal value is 32767,99998.

If the value is stored correct into the display it will be converted to a string as follows.

Example: PREFIX_NONE

Value:	0x058F03A6

String:	“1423,934”

Avoid sending decimals in case of ERROR type.

NOTE: This value can have additional numerical boundaries.

VOLTAGE, CURRENT, POWER, TEMPERATURE and PROCENT

This value type is used in same manner as PREFIX_NONE. The conversion is the same but the protocol is adding the prefix.

Example: VOLTAGE

Value:	0x058F03A6

String:	“1423,934V”

NOTE: This value can have additional numerical boundaries.

OPERATION

This type is used for indication of operation level, failure level and battery operation status. To set an OPERATION a value must be set in the 32-bit value. Following table shows the values and their meaning:

| Function Operation State | Value (HEX) |
| --- | --- |
| OS_OFF | 0 |
| OS_WAKEUP | 1 |
| OS_READY_TO_START | 2 |
| OS_STARTING | 3 |
| OS_STOPPING | 4 |
| OS_ON | 5 |
| Function Operation State | Value (HEX) |
| FL_OK | 10 |
| FL_WARNING | 11 |
| FL_SIMPLE_FAILURE | 12 |
| FL_EMPTY | 13 |
| FL_CRITICAL_FAILURE | 14 |
| Battery Operation State* | Value (HEX) |
| BS_STANDBY | 20 |
| BS_DISCHARGING | 21 |
| BS_CHARGING | 22 |

The states are controlled by the LPS except for “Battery Operation State”

If the correct value is stored into the display, it will show a string as follows:

Example: OPERATION

Value: 0x00000003

String: “Power Down”

LANGUAGE

This value is used for language setting of the interface. This value is set by the LPS. To set a LANGUAGE a value must be set in the 32-bit value. Following table shows the values and their meaning:

| LANGUAGE | Value (HEX) |
| --- | --- |
| ENG | 0 |
| DK | 1 |

Example: LANGUAGE

Value: 0x00000001

String: “Danish”

SERIAL

This value contains a serial like unit serial. The value uses the complete area of the 32-bit value. The LPS must send the serial in following format:

Example: SERIAL

Value (DEC): 2001001234

String: “200100-1234”

VERSION

This value contains a version number like software version number. The value uses the complete area of the 32-bit value. The LPS must send the serial in following format:

Example: VERSION

Value (DEC): 000102

String: “00.01.02”

DATE

This value contains a date like production date. The value uses the complete area of the 32-bit value. The LPS must send the serial in following format:

Example: DATE

Value (DEC): 20160617

String: “2016-06-17”

TIME

This value is used to store time. The two MSB of the 32-bit value are containing the minutes while the two LSB are containing the seconds.

Example: TIME

Value: 0x0053000F

String: “01:23:15”

If the time value is only presented by minutes and the two LSB are not used, the format will be:

Value: 0x00530000

String: “01:23”

The sign (-/+) of the value is read by the last bit of the MSB.

NOTE: This value can have additional numerical boundaries.